Written Assignment Unit 1

An Assignment submitted in partial fulfillment                                                                                            of the requirements for the degree of

Bachelor of Science

in

Computer Science

CS 2203 – Databases

Term 1 - 2024

University of The People

September 09, 2024

Connection-Oriented and Connectionless Protocols

A protocol refers to a set of rules controlling how computers communicate with each other. Network protocol functions include addressing, data packet format, segmentation (splitting long messages into small pieces), embedding control information, detecting errors, controlling data flow, and controlling connection (Salih, 2021). This paper provides a comparative analysis of connection-oriented and connectionless, which are the two primary types of network protocols. Connection-oriented protocols establish a connection between the sender and receiver before transmitting data, while connectionless protocols do not establish a connection before transmitting data (Iyanda, 2021). The comparison between these two protocols will be based on their advantages and disadvantages and design issues. Finally, the paper will present a recommendation to resolve one of the protocol’s design-related problems.

A connectionless protocol such as UDP is credited for having a lower overhead because it neither requires establishing nor maintaining connections, thus leading to lower overhead and faster transmission speeds. However, connection-oriented protocols require more overhead due to the need to establish and maintain connections, which can result in slower transmission speeds (Sheldon & Multimedia, 2001). Therefore, a connectionless protocol has a lower over, while a connection-oriented protocol has a higher overhead.

Moreover, connection-oriented protocols such as TCP ensure that data is delivered properly and in the correct order by establishing a connection between the sender and receiver before transmitting data. On the other hand, connectionless protocols do not guarantee data delivery, as they do not perform extensive error checking and packet acknowledgment. This could lead to packet loss or corruption, which might be unacceptable for certain applications that require high levels of data integrity (Sheldon & Multimedia, 2001). Therefore, connection-oriented protocols are much more reliable for delivering data as compared to connectionless protocols.

In addition, the time required to establish a connection in connection-oriented protocols can add latency to the communication, making it less suitable for real-time applications such as online gaming and voice over IP (VoIP). On the contrary, connectionless protocols are better suited for real-time applications that require low latency, such as online gaming and VoIP (Sheldon & Multimedia, 2001). Therefore, connection-oriented protocols are not suited for real-time applications while connectionless protocols.

Reading design issues, the connection-oriented protocols require a logical connection to be established before data exchange and maintained until communication ends. This setup and teardown process can introduce significant overhead, leading to increased latency. Secondly, during the connection establishment, resources are reserved for each connection, which can be inefficient if the resources are not fully utilized. Finally, due to the dedicated resources per connection, connection-oriented protocols may face scalability issues when handling a large number of simultaneous connections (Oracle Corporation, 2010). On the other hand, connectionless protocols do not guarantee delivery, order, or error-checking of data packets, which can lead to packet loss or out-of-order delivery. In addition, the absence of flow control in connectionless protocols can result in network congestion when senders overwhelm the network with data packets. Finally, connectionless protocols are more vulnerable to security threats, such as spoofing and denial-of-service attacks, as they lack built-in mechanisms for verifying the authenticity and integrity of data packets (Oracle Corporation, 2010). Therefore, both connection-oriented and connectionless protocols have design issues hindering their functionalities as expected by the users.

One possible solution to reduce the setup and teardown overhead in connection-oriented protocols is implementing connection pooling. Connection pooling is a technique in which a pool of pre-established connections is maintained and reused, rather than creating a new connection for each request. This approach significantly reduces latency by eliminating the need for repeated setup and teardown processes, allowing for faster and more efficient data exchange (Oracle Corporation, 2010). Therefore, implementing connection pooling can help to resolve the overhead issue in connection-oriented protocols.

References

Iyanda, J. (2021). Connection-oriented TCP vs connectionless UDP: A comparative analysis of network protocols. https://shorturl.at/OUzfM

Oracle Corporation. (2010). Connection-oriented and connectionless protocols. https://docs.oracle.com/cd/E19620-01/805-4041/6j3r8iu2f/index.html

Salih, H. K. (2021). Network protocols. https://www.researchgate.net/publication/357226205_Network_Protocols?channel=doi&linkId=61c23ca3abcb1b520ad49c8e&showFulltext=true

Sheldon T., & Multimedia, B., G. (2001). Connection-oriented and Connectionless services (Linktionary term). https://www.linktionary.com/c/connections.html