Discussion Assignment Unit 2

Some of the potential problems that can arise in Non-Return to Zero (NRZ) encoding when a continuous stream of 0s or 1s is transmitted include:

- Lack of Synchronization: In NRZ encoding, the primary issue is that the receiver relies on signal transitions (changes between 0 and 1) to synchronize its clock with the sender. A long run of continuous 0s or 1s means there are no transitions, leading to clock drift or misalignment (Satya Sai Lakshmi et al., 2020). If the receiver’s clock is slightly faster or slower, it can lose count of the number of bits, causing errors in interpreting the data

- Clock Recovery problem: Since there are no transitions during long streams of identical bits, the receiver cannot recover its clock signal, which is used to keep track of when each bit starts and ends (Satya Sai Lakshmi et al., 2020). This can result in misreading the length of a bit, causing data corruption.

- DC Bias: NRZ encoding does not inherently return to a zero voltage between bits, which can lead to a buildup of a DC component in the signal over time. Continuous streams of the same signal level (either all 0s or all 1s) exacerbate this problem, potentially causing issues with hardware that requires a zero average voltage.

- Indistinguishable idle state: Another issue is that NRZ encoding may fail to distinguish between long runs of 0s and an idle state. Without transitions, the receiver might interpret the lack of signal change as inactivity or an error, rather than valid data transmission.

- Increased error sensitivity: In the absence of transitions, any small noise or disturbance in the signal could be misinterpreted as a bit flip, leading to errors in decoding the data. Transitions help the receiver recalibrate, but a continuous stream lacks these checkpoints.

To mitigate these issues, other encoding schemes like NRZI, Manchester, or 4B/5B are often used, which introduce transitions or extra bits to ensure better synchronization​.

Reference

Dordal, P. (2023). An introduction to computer networks.

Satya Sai Lakshmi, K. A., Keerthi, A., Sri, K. M., & Vinodhini, M. (2020). Code with crosstalk avoidance and error correction for network on chip interconnects. 2020 4th International Conference on Trends in Electronics and Informatics (ICOEI)(48184). https://doi.org/10.1109/icoei48184.2020.9143002