Written Assignment Unit 2

An Assignment submitted in partial fulfillment                                                                                            of the requirements for the degree of

Bachelor of Science

in

Computer Science

CS 2204 – Communication and Networking

Term 1 - 2024

University of The People

September 19, 2024

- What is crosstalk? How is it minimized in case of a twisted pair of wire?

Crosstalk is the interference that occurs between adjacent wires during data transmission, causing undesirable effects in another circuit or channel (Lakshmi at., 2020). It is caused by either parasitic capacitance or inductance and can result in timing violations, extra power dissipation, logical errors, and bit corruption (Lakshmi at., 2020). In Network on Chip (NoC) communication fabrics, crosstalk is an issue that can affect the reliable transfer of data between cores. Crosstalk in twisted pair cables can be minimized by increasing the physical distance between cable pairs, which is often achieved by increasing the sheath thickness or using specific shapes (Cadence System Analysis, 2023). However, this method can reduce cable flexibility and duct utilization. Another effective method is to increase the twisting frequency of the wires in unshielded cable pairs, which helps prevent interference crossover between twisted cable pairs (Cadence System Analysis, 2023). Additionally, using shielded twisted pair cables can significantly reduce crosstalk compared to unshielded twisted pairs.

- Why are two separate frequencies used for uplink and downlink transmission in case of satellite communication?

In satellite communication, the process of transmitting data from a ground station to a satellite is known as uplink transmission, while the process of transmitting data from a satellite to a ground station is known as downlink transmission. During uplink transmission, the ground station sends a signal to the satellite using a designated frequency band. The signal is then received by the satellite's antenna and processed by its onboard systems. The satellite then amplifies the signal and retransmits it back to Earth, using a different frequency band to avoid interference with the uplink signal (Li et al., 20201). On the other hand, during downlink transmission, the satellite sends a signal to the ground station using a designated frequency band. The signal is then received by the ground station's antenna and processed by its systems. The ground station then amplifies the signal and retransmits it to its intended recipients, using a different frequency band to avoid interference with the downlink signals. Both uplink and downlink transmissions are critical components of satellite communication, enabling a wide range of applications such as television broadcasting, internet connectivity, and remote sensing (Li et al., 20201) However, downlink transmission is often more challenging due to the longer distance between the satellite and the ground station, as well as the need to transmit data to multiple recipients simultaneously (Li et al., 20201). To overcome these challenges, advanced technologies such as multiple-input multiple-output (MIMO) systems and frequency reuse schemes are often employed in downlink transmission.

- Differentiate between the 2 switching techniques (Circuit Switching and Packet Switching).(4 differences)

Circuit Switching and Packet Switching are two different methods of transmitting data over a network. Here are four differences between the two:

- Connection type: Circuit switching requires a dedicated physical path between the two communicating devices for the entire duration of the communication. In contrast, packet switching does not require a dedicated path; data is broken into packets and sent over different paths to the destination, where they are reassembled (Murthy, 2009).

- Resource allocation: In circuit switching, resources (bandwidth, time slots) are reserved for the duration of the connection, even if no data is being transmitted. In packet switching, resources are allocated on a per-packet basis, allowing for more efficient use of network resources (Murthy, 2009).

- Quality of service (QoS): Circuit switching provides a guaranteed QoS as the dedicated path ensures consistent data transmission. Packet switching does not provide a guaranteed QoS as packets may experience varying levels of delay, jitter, and loss depending on network conditions (Murthy, 2009).

- Setup time: Circuit switching requires a significant amount of setup time to establish the dedicated path before data transmission can begin. Packet switching has minimal setup time as data can be sent as soon as the first packet is ready to be transmitted (Murthy, 2009).

- What is the answer to Exercise 2.7 from Section 2 of our textbook with this change: the 4th transmission (B sends to D) does not occur?

In the modified Exercise 2.7, where the 4th transmission (B to D) does not occur:

- A sends to D: All switches flood the packet, and S1, S2, S3, and S4 learn A's location.

- D sends to A: S2 forwards directly to S1, and no flooding occurs. S3 and S4 do not learn D’s location.

- A sends to B: All switches flood the packet again, but no new information is gained about B.

Without the 4th transmission, switches S2 and S4 never learn B's location. As a result, only S1 and S2 know about A and D, while S3 and S4 only know A’s location.

References

Lakshmi, K. A. S. S., Keerthi, A. M., Sri, K. M., & Vinodhini, M. (2020, June). Code with crosstalk avoidance and error correction for network on chip interconnects. In 2020 4th International Conference on Trends in Electronics and Informatics (ICOEI)(48184) (pp. 75-79). IEEE.

Cadence System Analysis. (2023, August 11). Reducing crosstalk in data communication. https://resources.system-analysis.cadence.com/blog/msa2022-reducing-crosstalk-in-data-communication

Murthy, C.S.V. (2009). Data communication and networking. Global Media. ProQuest Ebook Central.

Li, K. X., You, L., Wang, J., Gao, X., Tsinos, C. G., Chatzinotas, S., & Ottersten, B. (2021). Downlink transmit design for massive MIMO LEO satellite communications. IEEE Transactions on Communications, 70(2), 1014-1028.