Answer the following questions in your own words:

QUES 1 - What do you understand by The Hidden-Node Problem? Give an example of a three-sender hidden-node problem? Describe 2 different spatial configurations which exemplify this issue.

ANS - The problem of a node not being able to detect a potential competitor for the medium because the competitor is too far away is called the hidden node problem.

The three sending nodes can be laid out at the vertices of an equilateral triangle, with the receiving node in the center.

Alternatively, with impermeable walls the following arrangement works:

sender1

------------

receiver 	 sender2

------------

sender3

QUES 2 - What is the 4B/5B encoding for the 3-byte (ASCII) string “Sun”?

ANS – 0101110101 0111101011 0111011100

QUES 3 - What is the 16-bit Internet checksum for the 3-byte binary message 00110001 00110010 00110011?

ANS – 01100100 00110010 which should have the bits inverted to 10011011 11001101.

Note: both answers should be considered ok for the purposes of the WA assessment.

QUES 4 - Suppose a message is 11011000. What is the CRC checksum using the polynomial corresponding to 1011 to give a 3-bit answer? (Show the solution steps, not just the answer).

ANS –

Doing modulo-2 division of 11011000 (with 3 0s added at the end) by 1011, as shown below, yields a remainder of 011. The latter is the CRC checksum.

____________
1011| 11011000000
      1011
       1101
       1011
        1100 
        1011
         1110 
         1011
          1010 
          1011
           001000

1011

011

QUES 5 - What is Time Domain Reflectometry?

ANS - A Pulse of known shape is injected into the cable. If the pulse hits an obstacle or the end of the cable, an echo well be generated and sent back. By carefully timing the interval between sending the pulse and receiving the echo, it is possible to localize the origin of the echo. This technique is known as Time Domain Reflectometry.

It is used to detect cable breaks, excessive length of cables, bad taps, or loose connectors.