Learning Journal Unit 3

An Assignment submitted in partial fulfillment                                                                                            of the requirements for the degree of

Bachelor of Science

in

Computer Science

CS 2204 – Communication and Networking

Term 1 - 2024

University of The People

September 26, 2024

- Give any two reasons why networks might use Error Correcting Codes instead of Error Detection and Retransmission.

Error Correcting Codes (ECC) and Error Detection and Retransmission are two methods used in networking to handle errors that may occur during data transmission. While both methods aim to ensure data integrity, there are situations where Error Correcting Codes might be preferred over Error Detection and Retransmission. Here are two reasons why:

1. High error probability environments: In environments where errors are likely to occur, such as in wireless communications or over long-distance transmissions, using Error Correcting Codes can be more efficient. These codes add redundancy to the data, allowing the receiver to correct errors without needing to request retransmission. This can significantly reduce the time and bandwidth that would otherwise be spent on retransmitting data, improving the overall performance and reliability of the communication system.

2. High latency networks: In networks with high latency, such as satellite links, the time it takes to detect an error, request retransmission, and receive the corrected data can be substantial. Error Correcting Codes can help mitigate this issue by allowing the receiver to correct errors locally, without the need to wait for a retransmission. This can greatly reduce the overall delay in communication, making it more suitable for real-time applications or for scenarios where latency is a critical factor.

In conclusion, while Error Detection and Retransmission can be effective in many scenarios, Error Correcting Codes offer advantages in terms of efficiency and latency reduction in certain environments, making them a valuable tool in networking.

Bibliography

Peterson L., & Bruce D. (2024). 2.4 error detection — Computer networks: A systems approach version 6.2-dev documentation. ttps://book.systemsapproach.org/direct/error.html