Learning Journal Unit 5

An Assignment submitted in partial fulfillment                                                                                            of the requirements for the degree of

Bachelor of Science

in

Computer Science

CS 2204 – Communication and Networking

Term 1 - 2024

University of The People

October 10, 2024

Questions: What is the fundamental difference between a Layer 2 switch and a Layer 3 router?

Layer 2 Switch:

A Layer 2 switch operates at the data link layer (Layer 2) of the OSI model. It forwards data based on MAC addresses. When a device sends a data packet, the Layer 2 switch looks at the destination MAC address and forwards the packet to the correct port that connects to the recipient device. This process is known as hardware switching. Layer 2 switches are often used within local area networks (LANs) to connect devices within the same network segment and provide high-speed, low-latency data transfer. However, Layer 2 switches cannot route traffic between different networks or VLANs, as they only operate within a single network segment or broadcast domain​ (ManageEngine, 2024).

Layer 3 Router:

A Layer 3 router, on the other hand, operates at the network layer (Layer 3) and forwards data based on IP addresses. Routers handle traffic between different network segments (such as subnets or VLANs) and often between different WANs (wide-area networks). Layer 3 devices also support routing protocols like RIP or OSPF to dynamically find the best paths for data to travel across networks. Layer 3 routers create separate broadcast domains and allow for inter-VLAN routing, enabling communication between different network segments.

While Layer 3 switches can perform basic routing functions similar to routers, such as inter-VLAN routing, routers offer more advanced routing capabilities, including support for multiple routing protocols and connections to wide-area networks (WANs)​ (ManageEngine, 2024). Thus, Layer 2 switches are best suited for simpler, smaller networks, while Layer 3 routers or switches are required for larger, more complex network infrastructures.

Reference

ManageEngine, C. (2024). Switch Port & IP address management software by ManageEngine OpUtils. ManageEngine OpUtils. https://www.manageengine.com/products/oputils/tech-topics/layer2-vs-layer3-switch.html