Discussion Assignment Unit 6

Introduction to UDP and stop-and-wait protocol

In the proposed application-level protocol, the client retrieves a file from a remote server using UDP. The client sends a file name to the server, and the server responds with a series of data packets containing the file. The protocol employs a stop-and-wait mechanism to ensure reliability and sequence in data delivery. However, using UDP, typically an unreliable protocol, raises certain issues that need to be addressed, especially regarding process crashes.

Potential problems with crashing processes

One significant concern with this protocol is what happens when either the client or the server crashes. UDP does not inherently provide mechanisms for detecting lost packets or ensuring data delivery. If a server crashes mid-transfer, the client might wait indefinitely for the next packet, leading to a timeout situation. Conversely, if the client crashes, the server, unaware due to UDP's stateless nature, might continue sending packets that are never received. This lack of awareness and acknowledgment can result in an incomplete file transfer, with the client missing parts of the file if the server cannot resume from the last acknowledged packet.

Ensuring reliability

To ensure reliable communication over UDP, additional protocol layers need to be implemented. The stop-and-wait protocol attempts to address this by ensuring that the client acknowledges each packet before the next is sent. While this adds a layer of reliability, it does not resolve the issue of process crashes. A possible solution could involve implementing a failover mechanism, where the client or server can detect a crash and either restart the session or continue from the last acknowledged packet. This would require maintaining state information, a concept not native to UDP.

Sequencing and data integrity

Sequence numbers, already a part of the stop-and-wait protocol, would help ensure data packets are processed in the correct order. However, additional checks, such as checksums, should be introduced to verify data integrity, ensuring that packets are neither duplicated nor corrupted during the transfer process.

Discussion question:

How can we handle crashes in the client or server processes using the stop-and-wait protocol? What mechanisms can ensure the client can resume file retrieval if a crash occurs after the request is sent?

Reference

Dordal, P. (2023). An introduction to computer networks.