Written Assignment Unit 6

An Assignment submitted in partial fulfillment                                                                                            of the requirements for the degree of

Bachelor of Science

in

Computer Science

CS 2204 – Communication and Networking

Term 1 - 2024

University of The People

October 16, 2024

Written Assignment Unit 6

Fragmentation of IPv4 Packets

When transferring 2000 bytes of user data with a single UDP send, it is essential to consider the Maximum Transmission Unit (MTU) of the network, which is typically 1500 bytes for Ethernet. The IPv4 header is 20 bytes, and the UDP header is 8 bytes, totaling 28 bytes for headers (Peterson & Davie, 2020). This means for a 2000-byte data payload, only 1472 bytes can be encapsulated in one IP packet after accounting for headers.

Thus, the 2000 bytes need to be split into two fragments:

- First Fragment: Contains 1472 bytes of user data, plus 28 bytes for the headers, resulting in a total of 1500 bytes.

- Second Fragment: Contains the remaining 528 bytes of user data and 20 bytes of an IP header (the UDP header is not needed in subsequent fragments), totaling 548 bytes.

This results in two fragments to successfully transmit the 2000 bytes.

Problems with Remote Procedure Call (RPC)

Despite its conceptual elegance, Remote Procedure Call (RPC) faces several challenges:

- Latency and performance: RPC often abstracts away network latency, giving a false sense of the immediacy usually associated with local function calls (Peterson & Davie, 2020). This abstraction can lead to performance issues when latency is unexpectedly high, as the network needs time to establish connections and transmit data.

- Complexity in error handling: While RPC simplifies communication, it does not inherently offer robust error handling (Peterson & Davie, 2020). Network failures, server crashes, or timeouts need to be managed explicitly by the developer, complicating the development process.

- Security concerns: RPC can potentially expose vulnerabilities due to its nature of executing code across networks (Peterson & Davie, 2020). Ensuring secure and authenticated communications between client and server involves additional layers of complexity and can lead to potential security risks if not properly managed.

Importance of timestamping in real-time applications

In the context of Real-time Transport Protocol (RTP), timestamping is crucial for maintaining synchronization and order in real-time applications, such as audio and video streaming. Timestamping allows receivers to play media streams at the correct pace and order, despite variations in packet arrival times due to network delays (GeeksForGeeks, 2022). This is critical for maintaining the quality and seamlessness of real-time interactions, ensuring that media playback is both smooth and synchronized.

Purpose of UDP

UDP (User Datagram Protocol) exists because it provides a simple, connectionless way of sending messages, unlike TCP, which establishes a connection and ensures reliable delivery. UDP is beneficial for applications where speed and efficiency are prioritized over reliability, such as live broadcasts or online games (Peterson & Davie, 2020). Allowing user processes to send raw IP packets would increase complexity due to lack of basic error checking and would not provide a consistent layer of abstraction that UDP offers for handling datagrams.

How QUIC reduces RTTs

QUIC is designed to improve performance in secure web connections by eliminating multiple Round-Trip Times (RTTs) required to establish a connection. Unlike traditional TCP, which requires a handshake to establish a connection and an additional handshake for TLS security, QUIC combines these steps. It uses a single handshake for security and connection setup, reducing latency and speeding up session initiation (Dordal, 2023). This design significantly improves web page load times by minimizing the overhead introduced by multiple handshakes.

References

GeeksForGeeks. (2022, December 8). Real time transport protocol (RTP). https://www.geeksforgeeks.org/real-time-transport-protocol-rtp/

Peterson, L., & Davie, B. (2020). Computer networks: A systems approach. Morgan Kaufmann. https://book.systemsapproach.org/e2e/rtp.html

Dordal, P. (2023). An introduction to computer networks.