Understanding Simultaneous TCP Connection Termination

In the TCP/IP protocol, connections are typically established using a three-way handshake process involving a sequence of packet exchanges between the client and server. However, while simultaneous TCP connection initiations are rare due to the structured nature of the handshake, simultaneous connection termination, where both parties decide to close a connection at the same time, is a more common occurrence (Dordal, 2022). This situation typically arises when both nodes send a FIN (finish) packet to each other, indicating they wish to terminate the connection.

Negotiating Simultaneous FIN Packets

When both TCP nodes engage in the simultaneous sending of FIN packets, the process follows a specific negotiation sequence. Each node sends a FIN packet, prompting the opposing node to acknowledge this with an ACK packet. After sending the FIN packet, a node transitions to a CLOSE_WAIT state, and once it acknowledges the FIN from the other node, it moves into the LAST_ACK state. Meanwhile, each node will also enter a FIN_WAIT_1 state after sending its own FIN packet (GeeksforGeeks, 2022).

Ladder Diagram and State Transitions

A ladder diagram illustrating the simultaneous TCP connection termination would depict the exchange of packets as follows:

- Node A sends a FIN to Node B and enters the FIN_WAIT_1 state.

- Simultaneously, Node B sends a FIN to Node A and also enters the FIN_WAIT_1 state.

- Node A receives the FIN from Node B, sends an ACK, and transitions to the CLOSING state.

- Node B receives the FIN from Node A, sends an ACK, and transitions to the CLOSING state.

- Both nodes receive the ACK of their respective FIN packets and move to the TIMEWAIT state.

TIMEWAIT State

The TIMEWAIT state is crucial as it ensures that the final ACK has been received reliably and allows any delayed segments to pass through the network without mistakenly being interpreted as part of a new connection (TCP/IP Guide, 2005). Notably, in the case of simultaneous TCP connection termination, both nodes will enter the TIMEWAIT state since they have both initiated a FIN exchange and received an acknowledgment for it (Dordal, 2022). This ensures that each node appropriately finalizes the connection closure process.

In summary, while simultaneous initiations are rare due to the TCP handshake process, simultaneous termination allows both parties to gracefully close the connection by exchanging FIN packets. Understanding this negotiation process is crucial for network reliability and performance.

References

Dordal, P. L. (2022, March 26). 12: TCP transport. Engineering LibreTexts. https://eng.libretexts.org/Bookshelves/Computer_Science/Networks/Book%3A_An_Introduction_to_Computer_Networks_(Dordal)/12%3A_TCP_Transport

GeeksforGeeks. (2022, May 12). TCP connection termination. https://www.geeksforgeeks.org/tcp-connection-termination/

TCP/IP Guide. (2005, September 20). The TCP/IP guide - TCP connection termination. Welcome to The TCP/IP Guide!. https://www.tcpipguide.com/free/t_TCPConnectionTermination-4.htm