Project: Digital Dice Roller

Godfrey Ouma

University of the People

CS 1105-01: Digital Electronics & Computer Architecture

Afrah Nazir

July 11, 2024

Project: Digital Dice Roller

Introduction

This project involves designing a digital dice roller using sequential circuits, specifically utilizing LEDs, a push button, a 4017 IC decade counter, a 555 Timer IC, and a potentiometer to show the interaction between registers and counters. This practical scenario is chosen to help understand and apply the concepts of sequential circuits in a real-world application, ensuring a comprehensive grasp of designing and analyzing such circuits while considering their input/output behavior.

Justification for selecting the Digital Dice Roller

The digital dice roller is an excellent choice for this project due to its simplicity and educational value. It offers an opportunity to explore the fundamentals of sequential circuits, counters, and timers engagingly and interactively. The project involves basic components that are easy to understand and work with, making it ideal for those studying digital electronics. Additionally, the project shows practical applications of digital electronics, such as timing, counting, and state transitions, which are essential concepts in the field of digital electronics.

Circuit Design and Explanation

Components:

- LEDs: 6 LEDs representing dice numbers 1-6.

- Push Button: Starts and stops the flashing of LEDs.

- 4017 IC: Decade counter driving the LEDs.

- 555 Timer IC: Provides the clock pulses.

- Potentiometer (RV1): Adjusts the flashing speed of LEDs.

Design

In this digital dice circuit, 6 LEDs correspond to the numbers (1-6) on a dice. When the push button is pressed, the LEDs begin flashing and stop upon button release. The illuminated LED post-release indicates the dice result. For instance, if the fifth LED remains ON, it signifies a roll of 5. The circuit connects these LEDs to outputs Q0 to Q5, while the seventh output, Q6, loops back to RESET PIN 15. This setup ensures that after LED 6, the sequence restarts from the first LED at Q0 (Jayant, 2015). Figure 1 shows the circuit diagram used to implement the digital dice roller.

Figure 1

Digital Dirce Roller Circuit Diagram

Source: Jayant, 2015.

To activate the clock pulse at PIN 14 of the 4017 IC, an Astable mode 555 timer IC was employed. The oscillating output generated at PIN 3 of the 555 timer is directed to PIN 14 of the 4017, advancing the output with each clock pulse. The speed of the flashing LEDs can be adjusted using potentiometer RV1; rotating its knob changes the oscillation frequency of the 555 timer and thus the clock pulse rate. The frequency (F) of the 555 timer can be calculated using the formula: F = 1.44 / ((R1 + 2 * RV1) * C1).

Utilization of Registers and Counters in the project

Counters

The 4017 IC is a decade counter, which means it can count from 0 to 9 in binary form (Build Electronic Circuits, 2023). In this digital dice roller project, the 4017 IC is configured to count from 0 to 5, corresponding to the six faces of a dice. The 4017 IC advances its output from Q0 to Q9 sequentially with each clock pulse received at its clock input (PIN 14). Each of these outputs represents a specific state of the counter. In this project, only outputs Q0 to Q5 are used to represent the numbers 1 to 6 on the dice. The outputs Q0 to Q5 are connected to six LEDs. When the counter reaches Q6, it sends a signal back to the RESET pin (PIN 15), causing the counter to reset and start from Q0 again. This setup creates a cyclical count from 0 to 5.

Clock Pulse and Frequency Control:

The clock pulses required to drive the counter are provided by a 555 timer IC configured in Astable mode. This mode allows the 555 timer to generate a continuous square wave signal. The frequency of these clock pulses determines how fast the counter advances through its states. The potentiometer (RV1) connected to the 555 timer allows adjustment of this frequency, thus controlling the speed at which the LEDs flash. The formula F = 1.44 / ((R1 + 2 * RV1) * C1) is used to calculate the frequency of the 555 timer. By adjusting RV1, the resistance changes, which in turn changes the frequency of the oscillations.

Output Behavior:

When the push button is pressed, the 555 timer starts generating clock pulses that advance the counter. This causes the LEDs connected to the outputs Q0 to Q5 of the 4017 IC to flash sequentially. Releasing the push button stops the clock pulses, and the LED corresponding to the current state of the counter remains lit. This illuminated LED indicates the dice roll result.

Registers

Registers are used in digital circuits to store and hold data. In this project, while traditional registers (like those made from flip-flops) are not explicitly used, the 4017 IC functions similarly by holding the current state of the counter. The 4017 IC "registers" the current state of the dice roll. When the clock pulses stop (when the push button is released), the counter holds its current state. This state is represented by one of the LEDs (Q0 to Q5) being lit. The state remains stable and unchanged until the next set of clock pulses is initiated by pressing the push button again. This mechanism ensures that the dice roll result is held and displayed until a new roll is initiated, mimicking the behavior of a register that holds a value until it is updated.

Summarized Overview of Project's Key Components and Their Interaction

1. 555 Timer IC (Astable Mode):

- Function: Generates continuous clock pulses at a controllable frequency.

- Role in Circuit: Provides the timing signal that drives the counter (4017 IC).

- Interaction: The output from PIN 3 of the 555-timer is connected to the clock input (PIN 14) of the 4017 IC. The frequency of the clock pulses is adjusted using a potentiometer (RV1), which changes the rate at which the LEDs flash.

2. 4017 IC (Decade Counter):

- Function: Acts as a counter that cycles through its outputs sequentially with each clock pulse.

- Role in Circuit: Drives the LEDs to represent the dice numbers 1 to 6.

- Interaction: The 4017 IC receives clock pulses from the 555-timer at its clock input (PIN 14). The outputs Q0 to Q5 of the 4017 IC are connected to six LEDs, each representing a number on the dice. The output Q6 is connected to the reset pin (PIN 15) to restart the count after reaching Q5.

3. LEDs:

- Function: Visual indicators representing the numbers on the dice.

- Role in Circuit: Display the current count/state of the 4017 IC.

- Interaction: Six LEDs are connected to the outputs Q0 to Q5 of the 4017 IC. When the push button is pressed, the LEDs flash sequentially, and the LED corresponding to the current state of the counter remains lit when the button is released.

4. Push Button:

- Function: Starts and stops the generation of clock pulses.

- Role in Circuit: Controls when the LEDs start flashing and when they stop, indicating the dice roll result.

- Interaction: Pressing the push button allows the 555-timer to generate clock pulses, advancing the 4017 IC counter. Releasing the button stops the clock pulses, and the currently lit LED indicates the dice roll result.

5. Potentiometer (RV1):

- Function: Adjusts the frequency of the clock pulses generated by the 555-timer.

- Role in Circuit: Controls the speed at which the LEDs flash.

- Interaction: Connected to the 555-timer circuit, the potentiometer changes the resistance, which in turn adjusts the oscillation frequency of the 555-timer. This affects the rate of clock pulses sent to the 4017 IC.

How They Work Together

- Clock Pulse Generation: The 555 timer IC in A stable mode generates continuous clock pulses (Campbell, 2021). The frequency of these pulses is adjustable via the potentiometer (RV1), allowing control over the speed of the dice roll simulation.

- Counting and Display: The clock pulses from the 555 timers drive the 4017 IC, which acts as a counter. As the counter advances with each pulse, it sequentially activates the LEDs connected to its outputs Q0 to Q5. Each LED represents a number on the dice (1-6).

- User Interaction: The push button initiates and stops the clock pulses. When pressed, the LEDs flash rapidly, simulating the rolling of a dice. Upon release, the clock pulses stop, and the currently lit LED indicates the result of the dice roll.

- Reset Mechanism: The connection from Q6 to the reset pin (PIN 15) on the 4017 IC ensures that the counter resets after reaching the sixth state, creating a continuous loop from 1 to 6.

Conclusion

This digital dice roller project effectively shows the principles of sequential circuits, registers, and counters. By analyzing and designing this circuit, students gain practical knowledge and skills in creating and understanding digital electronic systems. The integration of the 555 Timer and 4017 Decade Counter highlights how timing and counting can be used to produce interactive and logical output behaviors, providing a comprehensive learning experience.

# References

Build Electronic Circuits. (2023, May 23). CD4017 - A decade counter with decoded output. https://www.build-electronic-circuits.com/4000-series-integrated-circuits/ic-4017/

Campbell, S. (2021, November 14). 555 timer basics - Astable mode. Circuit Basics. https://www.circuitbasics.com/555-timer-basics-astable-mode/

Jayant. (2015). Digital dice circuit. Circuit Digest - Electronics Engineering News, Latest Products, Articles and Projects. https://circuitdigest.com/electronic-circuits/digital-dice-circuit-using-ic-555