Project: Digital Dice Roller

a. Scenario Selection

Scenario: Digital Dice Roller

Justification: This scenario is chosen for its simplicity and practical application in teaching fundamental concepts of sequential circuits, registers, and counters. The project involves creating a digital dice roller using LEDs to display numbers 1-6, a push button to control the rolling, a 4017 decade counter IC to drive the LEDs, a 555 timer IC to generate clock pulses, and a potentiometer to adjust the speed of the LED flashing. This project provides hands-on experience with commonly used components and helps students understand the collaborative behavior of registers and counters in real-world applications.

b. Step-by-Step Analysis of Circuit Design

1. Components and Connections:

- LEDs (D1-D6): Connected to the output of the 4017 decade counter IC to represent dice numbers 1-6.

- Push Button (S1): Used to start and stop the flashing of LEDs. It controls the enable pin of the 555 timer IC.

- 4017 Decade Counter IC: Drives the LEDs. It has ten outputs, but only the first six are used in this project.

- 555 Timer IC: Configured in astable mode to provide continuous clock pulses to the 4017 IC.

- Potentiometer (RV1): Connected to the 555 timer IC to adjust the frequency of the clock pulses, thus changing the flashing speed of LEDs.

2. Circuit Diagram:

- The 555 timer IC is configured in astable mode with the potentiometer adjusting the timing intervals.

- The output of the 555 timer (pin 3) is connected to the clock input of the 4017 decade counter (pin 14).

- LEDs are connected to the first six outputs of the 4017 IC (Q0-Q5), with current-limiting resistors in series.

- The push button is connected to the reset pin of the 555 timer to start and stop the clock pulses.

3. Operation:

- When the push button is pressed, the 555 timer starts generating clock pulses.

- These clock pulses are fed into the 4017 decade counter, causing its outputs to cycle through the connected LEDs.

- The LEDs flash sequentially, simulating the rolling of a dice.

- Releasing the push button stops the clock pulses, and the LEDs stop flashing, displaying a random dice number.

c. Utilization of Registers and Counters

- 555 Timer IC: Acts as a pulse generator (clock source) for the counter. The adjustable frequency via the potentiometer simulates different rolling speeds of the dice.

- 4017 Decade Counter IC: Utilizes its internal registers to count the clock pulses from the 555 timer. Each pulse increments the counter, and the current count is displayed on the LEDs.

d. Overview of Key Components

- LEDs: Indicate the current state of the counter, representing dice numbers 1-6.

- Push Button: Provides user control to start and stop the dice rolling.

- 4017 Decade Counter IC: Core component that counts the clock pulses and controls the LEDs based on the current count.

- 555 Timer IC: Generates the clock pulses required for the 4017 IC. The frequency of these pulses can be adjusted using the potentiometer, affecting the speed of LED flashing.

- Potentiometer (RV1): Allows the user to adjust the frequency of the 555 timer, simulating different rolling speeds.

This project effectively demonstrates the practical application of sequential circuits, registers, and counters. Students learn how these components interact to create a functional digital system, reinforcing their theoretical knowledge with hands-on experience.